App.py

import cv2

import logging

import socketserver

import threading

from threading import Condition

from http import server

# =========================

# HTML PAGE

# =========================

PAGE = """\

<html>

<head>

<title>Surveillance - USB Camera</title>

</head>

<body>

<center><h1>Live Stream</h1></center>

<center>

<img src="/stream.mjpg" width="640" height="480">

</center>

</body>

</html>

"""

# =========================

# FRAME BUFFER

# =========================

class StreamingOutput:

def __init__(self):

self.frame = None

self.condition = Condition()

def update(self, frame):

with self.condition:

self.frame = frame

self.condition.notify_all()

output = StreamingOutput()

# =========================

# HTTP HANDLER

# =========================

class StreamingHandler(server.BaseHTTPRequestHandler):

def do_GET(self):

if self.path == '/':

self.send_response(301)

self.send_header('Location', '/index.html')

self.end_headers()

elif self.path == '/index.html':

content = PAGE.encode('utf-8')

self.send_response(200)

self.send_header('Content-Type', 'text/html')

self.send_header('Content-Length', len(content))

self.end_headers()

self.wfile.write(content)

elif self.path == '/stream.mjpg':

self.send_response(200)

self.send_header('Age', 0)

self.send_header('Cache-Control', 'no-cache, private')

self.send_header('Pragma', 'no-cache')

self.send_header(

'Content-Type',

'multipart/x-mixed-replace; boundary=FRAME'

)

self.end_headers()

try:

while True:

with output.condition:

output.condition.wait()

frame = output.frame

if frame is None:

continue

# MJPEG FRAME FORMAT (VERY IMPORTANT)

self.wfile.write(b'--FRAME\r\n')

self.wfile.write(b'Content-Type: image/jpeg\r\n')

self.wfile.write(

b'Content-Length: ' + str(len(frame)).encode() + b'\r\n\r\n'

)

self.wfile.write(frame)

self.wfile.write(b'\r\n')

self.wfile.flush()

except Exception as e:

logging.warning(

'Streaming client disconnected %s: %s',

self.client_address, str(e)

)

else:

self.send_error(404)

self.end_headers()

# =========================

# THREADED SERVER

# =========================

class StreamingServer(socketserver.ThreadingMixIn, server.HTTPServer):

allow_reuse_address = True

daemon_threads = True

# =========================

# USB CAMERA THREAD

# =========================

def camera_thread():

cap = cv2.VideoCapture("/dev/video0", cv2.CAP_V4L2)

if not cap.isOpened():

print("âŒ Could not open USB camera")

return

cap.set(cv2.CAP_PROP_FRAME_WIDTH, 640)

cap.set(cv2.CAP_PROP_FRAME_HEIGHT, 480)

cap.set(cv2.CAP_PROP_FPS, 20)

cap.set(cv2.CAP_PROP_BUFFERSIZE, 1)

print("âœ… USB camera started")

while True:

ret, frame = cap.read()

if not ret:

continue

# Frame is already BGR â†’ encode directly

ret, jpeg = cv2.imencode(

'.jpg',

frame,

[cv2.IMWRITE_JPEG_QUALITY, 80]

)

if ret:

output.update(jpeg.tobytes())

# =========================

# MAIN

# =========================

if __name__ == "__main__":

logging.basicConfig(level=logging.INFO)

# Start camera thread

threading.Thread(target=camera_thread, daemon=True).start()

# Start HTTP server

address = ('', 8000)

server = StreamingServer(address, StreamingHandler)

print("USB Camera streaming at http://<PI_IP>:8000")

server.serve_forever()

iot_receive.py

import time

import telepot

import datetime

from telepot.loop import MessageLoop

TOKEN = "5982198371:AAEL3cD2U6IyORf9zlOD_sl5-X85MXyhKbc"

def handle(msg):

chat_id = msg['chat']['id']

# Ignore non-text messages

if 'text' not in msg:

return

command = msg['text']

now = datetime.datetime.now()  # get current time dynamically

print("Received:", command)

if command == '/status':

bot.sendMessage(chat_id, "Hi! This is the status message")

elif command == '/time':

bot.sendMessage(

chat_id,

f"Time: {now.hour}:{now.minute}:{now.second}"

)

elif command == '/date':

bot.sendMessage(

chat_id,

f"Date: {now.day}/{now.month}/{now.year}"

)

else:

bot.sendMessage(chat_id, "Unknown command")

bot = telepot.Bot(TOKEN)

MessageLoop(bot, handle).run_as_thread()

print("Bot is running...")

while True:

time.sleep(10)

iot_send.py

import telepot

token = '5982198371:AAEL3cD2U6IyORf9zlOD_sl5-X85MXyhKbc' # telegram token

receiver_id = 819800434

bot = telepot.Bot(token)

bot.sendMessage(receiver_id, 'This is a automated test message.',"")

ir sensor.py

import time

import RPi.GPIO as GPIO

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

sw=21

GPIO.setup(sw,GPIO.IN,pull_up_down=GPIO.PUD_UP)

a=0

while(1):

swstate = GPIO.input(sw)

if(swstate == False):

#MOTOR RIGHT DIRECTION

print("button pressed")

a=a+1

print(a+1)

time.sleep(0.5)

motor.py

import RPi.GPIO as GPIO

import time

#WARNING IGNOR AND BCM PIN MODE

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

# MOTOR PIN DECLAR

m1_0 = 6

m1_1 = 13

m2_0 = 19

m2_1 = 26

GPIO.setup(m1_0,GPIO.OUT)

GPIO.setup(m1_1,GPIO.OUT)

GPIO.setup(m2_0,GPIO.OUT)

GPIO.setup(m2_1,GPIO.OUT)

while True:

#MOTOR FORWARD DIRECTION

GPIO.output(m1_0,GPIO.HIGH)

GPIO.output(m1_1,GPIO.LOW)

GPIO.output(m2_0,GPIO.HIGH)

GPIO.output(m2_1,GPIO.LOW)

time.sleep(0.2)

# #MOTOR STOP DIRECTION

GPIO.output(m1_0,GPIO.LOW)

GPIO.output(m1_1,GPIO.LOW)

GPIO.output(m2_0,GPIO.LOW)

GPIO.output(m2_1,GPIO.LOW)

time.sleep(0.8)

# #MOTOR REVERSE DIRECTION

# GPIO.output(m1_0,GPIO.LOW)

# GPIO.output(m1_1,GPIO.HIGH)

# GPIO.output(m2_0,GPIO.LOW)

# GPIO.output(m2_1,GPIO.HIGH)

# #MOTOR LEFT DIRECTION

# GPIO.output(m1_0,GPIO.HIGH)

# GPIO.output(m1_1,GPIO.LOW)

# GPIO.output(m2_0,GPIO.LOW)

# GPIO.output(m2_1,GPIO.LOW)

# #MOTOR RIGHT DIRECTION

# GPIO.output(m1_0,GPIO.LOW)

# GPIO.output(m1_1,GPIO.LOW)

# GPIO.output(m2_0,GPIO.HIGH)

# GPIO.output(m2_1,GPIO.LOW)

# #MOTOR STOP DIRECTION

# GPIO.output(m1_0,GPIO.LOW)

# GPIO.output(m1_1,GPIO.LOW)

# GPIO.output(m2_0,GPIO.LOW)

# GPIO.output(m2_1,GPIO.LOW)

# time.sleep(1)

Sound sensor.py

import time

import RPi.GPIO as GPIO

import time

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

sw=20

GPIO.setup(sw,GPIO.IN,pull_up_down=GPIO.PUD_UP)

a=0

while(1):

swstate = GPIO.input(sw)

if(swstate == False):

#MOTOR RIGHT DIRECTION

print("button pressed")

a=a+1

print(a+1)

time.sleep(0.5)

ultrasonic.py

import RPi.GPIO as GPIO

import time

TRIG = 23

ECHO = 18

GPIO.setmode(GPIO.BCM)

GPIO.setup(TRIG, GPIO.OUT)

GPIO.setup(ECHO, GPIO.IN)

GPIO.output(TRIG, False)

time.sleep(2)

def get_distance():

GPIO.output(TRIG, True)

time.sleep(0.00001)

GPIO.output(TRIG, False)

timeout = time.time()

while GPIO.input(ECHO) == 0:

if time.time() - timeout > 0.02:

return None

pulse_start = time.time()

timeout = time.time()

while GPIO.input(ECHO) == 1:

if time.time() - timeout > 0.02:

return None

pulse_end = time.time()

duration = pulse_end - pulse_start

distance = (duration * 34300) / 2

return round(distance, 2)

try:

while True:

dist = get_distance()

if dist:

print(f"Distance: {dist} cm")

else:

print("Out of range")

time.sleep(0.5)

except KeyboardInterrupt:

GPIO.cleanup()

Driver app.py

import time

import random

import telepot

import threading

import RPi.GPIO as GPIO

#WARNING IGNOR AND BCM PIN MODE

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

#Ir_sensor

IR_Sensor=21

sound_sensor=20

#Ultrasound sensor

TRIG = 23

ECHO = 18

# MOTOR PIN DECLAR

m1_0 = 6

m1_1 = 13

m2_0 = 19

m2_1 = 26

token = '5982198371:AAEL3cD2U6IyORf9zlOD_sl5-X85MXyhKbc' # telegram token

#receiver_id = 969506549

receiver_id = 819800434

bot = telepot.Bot(token)

GPIO.setup(IR_Sensor,GPIO.IN,pull_up_down=GPIO.PUD_UP)

GPIO.setup(sound_sensor,GPIO.IN,pull_up_down=GPIO.PUD_UP)

GPIO.setup(TRIG, GPIO.OUT)

GPIO.setup(ECHO, GPIO.IN)

GPIO.setup(m1_0,GPIO.OUT)

GPIO.setup(m1_1,GPIO.OUT)

GPIO.setup(m2_0,GPIO.OUT)

GPIO.setup(m2_1,GPIO.OUT)

GPIO.output(TRIG, False)

time.sleep(2)

def forw():

#MOTOR FORWARD DIRECTION

print("Forward")

GPIO.output(m1_0,GPIO.HIGH)

GPIO.output(m1_1,GPIO.LOW)

GPIO.output(m2_0,GPIO.HIGH)

GPIO.output(m2_1,GPIO.LOW)

def reve():

print("Reverse")

#MOTOR REVERSE DIRECTION

GPIO.output(m1_0,GPIO.LOW)

GPIO.output(m1_1,GPIO.HIGH)

GPIO.output(m2_0,GPIO.LOW)

GPIO.output(m2_1,GPIO.HIGH)

def left():

print("Left")

#MOTOR LEFT DIRECTION

GPIO.output(m1_0,GPIO.HIGH)

GPIO.output(m1_1,GPIO.LOW)

GPIO.output(m2_0,GPIO.LOW)

GPIO.output(m2_1,GPIO.LOW)

def right():

print("Right")

#MOTOR RIGHT DIRECTION

GPIO.output(m1_0,GPIO.LOW)

GPIO.output(m1_1,GPIO.LOW)

GPIO.output(m2_0,GPIO.HIGH)

GPIO.output(m2_1,GPIO.LOW)

def stop():

print("Stop")

#MOTOR STOP DIRECTION

GPIO.output(m1_0,GPIO.LOW)

GPIO.output(m1_1,GPIO.LOW)

GPIO.output(m2_0,GPIO.LOW)

GPIO.output(m2_1,GPIO.LOW)

def random_move():

moves = [forw, reve, left, right]

move = random.choice(moves)

move()

time.sleep(random.uniform(0.3, 1.0))  # random movement time

stop()

def get_distance():

GPIO.output(TRIG, True)

time.sleep(0.00001)

GPIO.output(TRIG, False)

timeout = time.time()

while GPIO.input(ECHO) == 0:

if time.time() - timeout > 0.02:

return None

pulse_start = time.time()

timeout = time.time()

while GPIO.input(ECHO) == 1:

if time.time() - timeout > 0.02:

return None

pulse_end = time.time()

duration = pulse_end - pulse_start

distance = (duration * 34300) / 2

return round(distance, 2)

def distance_thread():

while True:

dist = get_distance()

# if dist:

#     print(f"Distance: {dist} cm")

if dist <5:

bot.sendMessage(receiver_id, 'Low Distance',"")

time.sleep(0.5)

# Create and start thread

dist_thread = threading.Thread(target=distance_thread, daemon=True)

dist_thread.start()

while True:

Irstate = GPIO.input(IR_Sensor)

soundstate = GPIO.input(sound_sensor)

if(Irstate == False or soundstate == False) :

random_move()

time.sleep(0.5)

else:

stop()